# 🧬 DNA-Link Protocol

A custom, fault-tolerant, bare-metal communication protocol for Arduino featuring hardware-level redundancy and recursive error correction.

DNA-Link is an experimental "bit-banging" data transmission protocol built from scratch. Instead of relying on standard hardware peripherals (like UART, SPI, or I2C) or heavy mathematical checksums (like CRC), it ensures data integrity through physical-layer redundancy (a "DNA-style" inverse bit check) and a highly efficient, infinitely recursive 8-bit error masking system.

## ✨ Key Features

- Custom Bit-Banging Engine: Complete manual control of clock synchronization and data lines using microsecond-level timing delays.

- Physical-Layer "DNA" Check: Transmits every bit alongside its inverse (e.g., 1 and 0). If line noise causes both pins to read the same state (e.g., 1 and 1), an error is instantly caught at the hardware level.

- Surgical Bitmask Correction: When corrupted bits are found, the receiver doesn't drop the packet. It dynamically generates an 8-bit mask (1 = corrupt, 0 = skip) and requests a re-transmission of only the broken bits.

- Infinite Recursion: The protocol uses a unified send_8bits() and recieve_8bits() architecture. If the error mask itself gets corrupted while being transmitted back to the sender, the Arduinos recursively swap roles to heal the mask before healing the actual data!

## 🛠️ Hardware Setup

### Requirements

- 2x Arduino Uno (or compatible microcontrollers)

- Jumper Wires

- Common Ground connection

### Pinout Guide

| Arduino Pin | Role | Description |
| --- | --- | --- |
| D13 | Clock (S1) | Synchronizes the transmission. Controlled by the active Sender. |
| D12 | Status (S2) | Acknowledgement line. HIGH = Data Good, LOW = Error Detected. |
| D6 | Data (X) | Carries the actual logical bit. |
| D5 | Inverse (X-Bar) | Carries the inverted bit (Logical NOT of D6) for the DNA check. |
| GND | Ground | CRITICAL: Both boards must share a common ground wire. |

Note: Roles are dynamic. The code automatically flips pin modes (INPUT/OUTPUT) on the fly when switching between sending data and performing error correction.

## 🧠 How the Architecture Works

### 1. The Physical Layer (Transmission)

The active Sender pulls the Clock line (D13) LOW to signal the start of a bit. It writes the data bit to D6 and the inverse to D5, holds for 50µs to let the electrical signals stabilize, and pulls the Clock HIGH.

### 2. The Verification Layer

The Receiver reads D6 and D5. Just like DNA base pairing, they must be opposites. If it reads 0 and 0 or 1 and 1, it logs the index of the corrupted bit and pulls the Status line (D12) LOW to alert the Sender.

### 3. The Recursive Correction Layer (The Magic)

If an error is flagged:

- Mask Generation: The Receiver generates an 8-bit array mapping the errors (e.g., [0,0,1,0,0,0,0,0] means the 3rd bit is broken).

- Role Swap: The Receiver temporarily becomes a Sender and transmits this 8-bit mask back down the wire.

- Healing: The original Sender reads the mask, loops through it, and re-transmits the correct bits wherever it sees a 1.

- Recursion: Because the mask is sent using the exact same underlying engine as the text data, any corruption of the mask triggers the exact same error-correction loop. The system will recursively dive as deep as necessary to ensure flawless data transfer.

## 🚀 How to Run It

- Wire up the two Arduinos according to the Pinout Guide.

- Upload the DNA_Link.ino code to both Arduinos (the code is identical for both sides).

- Open two instances of the Arduino IDE Serial Monitor (one for each COM port), set to 9600 baud.

- Initialize Sender: In one Serial Monitor, type @ and hit Enter. This board is now the Sender.

- Send Data: Type your message and hit Enter.

- Simulate a Fault: While a message is transmitting, momentarily pull out the D6 or D5 wire. You will see the protocol pause, enter correction mode, patch the missing bit, and print the flawless message on the Receiver's monitor!

## 👨‍💻 Author

Built by

Akshat Sarda

Het Thakkar

Kanhaiya Panjiyar

Shivansh Sahu